Remedial work completed- 75 Fourth Ave Avenues   February/March 2026

External

- Have specialist pest controllers check for borer – COMPLETED AND CERTIFICATE PROVIDED

- Check roof externally for rust and lifted laps. Remediate. Photos 4-5-6-9-10 FIXED

- Inspect roof internal for leakage or damaged timbers - remediate. Photo 51 FIXED

- Replace rusted iron on front porch. Photo 7 FIXED

- Fix right back corner of garage. FIXED

- New screws where some have missed purlin (in garage). Ph 12 FIXED

- Remediate and seal woodwork either side of the chimney. FIXED

Internal

- Replace skirting in ensuite toilet. FIXED

- Seal around vanity (front bedroom) FIXED

- Seal around shower in main bathroom. FIXED

Pest Specialist Inspection – 75 Fourth Ave Woodhill

A visual inspection of 75 Fourth Avenue on 13th February 2026 was completed for evidence of Borer activity.

The dwelling is a 1930s era bungalow constructed predominantly of Kauri timber.

At the time of inspection, the interior of the home was found to be in good condition with no visible concerns noted. The roof space was also inspected and found to be clear of any apparent issues. Insulation is currently installed.

In the subfloor area, evidence of borer activity was observed in some timber posts, joists, and bearers. Treatment is recommended to manage existing activity and reduce the risk of further infestation.

On the 17th February 2026, all exposed subfloor timbers were treated to control and prevent borer activity. Ongoing maintenance treatments at five year intervals are advised to ensure continued protection of the timber structure.

If there is anything more I can assist with, please feel free to reply.

Kind regards,

Shane Swann

Customer Service

Enviropro Pest Management

Phone: 09 4389227

106 Western Hills Drive, Kensington, Whangarei 0112

Web: www.enviropro.co.nz

Email: info@enviropro.co.nz