Remedial work completed- 75 Fourth Ave Avenues   Completed February/March 2026

External

- EnvironPro has done a pest controller check and treated under the house

- Surface rust on the roof has been prepped, treated and painted.

- Rotten purlins have been replaced

- 6 sheets of corrugated iron were replaced due to heavy surface rust and temporary patche

- All ridge capping has been re screwed into purlin. Ridge capping, new iron and all patches have been repainted.

- All lead head nails that needed replacing have been replaced.

- New weatherboard joiner and scribers next to the chimney.

- Remediate and seal woodwork either side of the chimney

Internal

- Fix and paint rotten skirting board in ensuite and seal around vanity

- Seal around the shower and fix towel rail to the wall.

- Reframe and reline back right corner in garage.

Pest Specialist Inspection – 75 Fourth Ave Woodhill

A visual inspection of 75 Fourth Avenue on 13th February 2026 was completed for evidence of Borer activity.

The dwelling is a 1930s era bungalow constructed predominantly of Kauri timber.

At the time of inspection, the interior of the home was found to be in good condition with no visible concerns noted. The roof space was also inspected and found to be clear of any apparent issues. Insulation is currently installed.

In the subfloor area, evidence of borer activity was observed in some timber posts, joists, and bearers. Treatment is recommended to manage existing activity and reduce the risk of further infestation.

On the 17th February 2026, all exposed subfloor timbers were treated to control and prevent borer activity. Ongoing maintenance treatments at five year intervals are advised to ensure continued protection of the timber structure.

If there is anything more I can assist with, please feel free to reply.

Kind regards,

Shane Swann

Customer Service

Enviropro Pest Management

Phone: 09 4389227

106 Western Hills Drive, Kensington, Whangarei 0112

Web: www.enviropro.co.nz

Email: info@enviropro.co.nz